Toolkit contents

1. Flow meter: measures the volumetric flow of brewery liquids over time (typically gallons or barrels [31 gallons per barrel] per min or hour). This can be used to determine liquid volume in tanks when the vessel may be closed or not have volume graduations.

2. Stopwatch: accurate device for measuring passage of time. Used in conjunction with the flow meter or multi meter. Can also be used to determine cycle time for processes.

3. Scale: used for measuring ingredients or waste products (spent yeast, hob trub, etc.) to determine side-streaming amounts. There are two scales included with the toolkit: one larger scale for bigger volume wastes like spent yeast, and a smaller scale to measure lower volume items such as hop oil, cleaning chemicals, etc.

4. Graduated bucket: used in conjunction with the scale to measure process waste.

5. Flow meter adapters: used with the flow meter to size up or down the connection size to ensure the flow meter can connect to most brewery pipes.

Optional: If you want to calculate peak demand from a motor/pump, you could (or ask an electrician to) use a multi-meter, which is a standard device for measuring voltage, amperage, or resistance of electrical circuits. We opted not to include or require use of a multi-meter due to safety concerns of the highly technical nature of using electrical equipment in equipment that by nature will be wet and poses a risk of electrocution.

Measuring process waste will help breweries identify waste that has the potential to be side-streamed, such as high BOD and COD items like hops and yeast that cause unnecessary stress on wastewater treatment facilities.

How to use a flow meter: (this is the specific flow meter included in the toolkit: https://www.gwkent.com/gpi-turbine-flow-meter-1-5-tc-1-turbine.html)

Installation: Mount your flow meter in a straight pipe section with sufficient space upstream and downstream of any bends, valves, or fittings. This eliminates turbulence that skews readings. Install with the flow direction arrow pointing correctly.

For liquid lines, mount horizontally or with flow upward to prevent air entrapment. Never install at the highest point of your piping where gas can collect.

Calibration & Setup: Zero your meter with static liquid in the line - no active flow, but with a full line. Set your totalizer to match batch requirements. Program density compensation if your meter requires it (specific gravity changes throughout fermentation affect some meter types).

Most modern meters have 4-20mA output signals. Calibrate your display/control system to read actual flow rates, not just electrical signals.

Operational Best Practices: Always purge air from lines before flow measurement begins - even small bubbles create erratic readings. Start flows gradually to avoid pressure surges that damage sensitive components.

Monitor for fouling, especially with high-protein worts or dry-hopped beers. Declining accuracy often indicates buildup on sensor elements.

For CIP operations, ensure your meter can handle cleaning chemicals and temperatures. Some electronic components need protection during aggressive cleaning cycles.

Troubleshooting: Erratic readings usually mean air entrainment, electrical interference, or fouling. Steady but incorrect readings suggest calibration drift or density changes you haven't compensated for.

Critical point: Flow meters measure volume, but brewers often need mass. Account for temperature and density variations, especially when transferring between vessels at different temperatures.

Measuring process waste with a graduated bucket:

Measuring process waste using a graduated bucket is one of the most fundamental skills for maintaining efficiency and quality control.

Equipment Setup: First, ensure you have a properly calibrated graduated bucket (I recommend at least 5-gallon capacity with clear measurement markings). Clean and sanitize it thoroughly - any residue will throw off your measurements and potentially contaminate samples.

Measurement Protocol:

Pre-Process Baseline: Before starting your brewing operation, record your initial volumes of wort, finished beer, or whatever liquid you're tracking. Document the starting temperature, as thermal expansion affects volume readings.

Collection Technique: Position your graduated bucket to catch all waste streams - spillage during transfers, trub separation, yeast harvesting during runoff, cleaning rinse water, and any rejected product. Don't forget the often-overlooked sources like sample pulls for testing and line purging.

Reading Accuracy: Always read measurements at eye level with the meniscus (the curved surface of the liquid). Read from the bottom of the curve for clear liquids, and the top for opaque ones. Take readings when the liquid has settled and isn't sloshing.

Temperature Compensation: Record the temperature of your waste liquid. Most graduated buckets are calibrated at 68°F (20°C). Significant temperature variations will affect volume accuracy by roughly 0.2% per 10°F difference.

Documentation: Log each measurement immediately with timestamps, batch numbers, and waste source identification. Calculate your waste percentage: (Total Waste Volume / Total Input Volume) × 100.

Quality Breweries target under 3% total process waste. If you're consistently above 5%, investigate your transfer procedures, equipment seals, and handling techniques.

The key is consistency in your measurement protocol - this data becomes invaluable for optimizing your brewing efficiency and identifying problem areas before they impact your bottom line.