# Topics for today:

- Wave interference

  - When waves arrive from different sources (or from a reflection), the wave pressures may be in phase or out of phase: the pressures may add or subtract from each other.

  - Interference can cause the sound to be louder or softer at different places in a room.

  - Interference can affect an audio recording depending upon the microphone location.

- Sound level and the decibel

  - Common sound pressures vary from tens of micropascals (very quiet) up to a few pascals (very loud).

  - It can be inconvenient to deal with this wide range of numbers (0.00002 to 2).

  - Customarily, acousticians convert to decibels (dB), so that the wide numerical range of pressure gets converted into a range from zero to 100.

  - The decibel for pressure is defined:   dB = 10 log10((pressure)2/(20 micropascals)2) .

  - This can be adjusted mathematically to be  dB = 20 log10((pressure)/(20 micropascals)) .

  - 20 micropascals is the “threshold of hearing” at frequencies for which the ear is most sensitive (2-3 kHz).  This is zero dB sound pressure level (SPL).

  - 2 pascals is very loud, almost painful, and corresponds to 100 dB SPL.

# Topics for the next lecture:

- Reading assignment:  Chapters 9, 10, and 12.

- Start anatomy and physiology of the ear.